Audio and Acoustics Test Software新功能介绍

The Audio and Acoustics Test Configurator has many features, including testing different types of microphones and speakers in one execution, different types of signals and measurements in one execution, combining parameter configuration and test execution in one application, displaying real-time test results during test execution, filtered test results and TestStand reports after test execution, and a real-time view of configured stimulus signals and parameters.?The Audio and Acoustics Test Configurator has the Configuration and Execution panes. The former can configure all the parameters needed for different audio and acoustics test sequences. The latter can run tests and check test results and report in the same way as you do in TestStand.?To configure test parameters:
1. Open the Audio and Acoustics Test Configurator in the C:\Program Files\National Instruments\Audio and Acoustics\Configurator directory.
2. Choose Configuration and then Microphone. Select a microphone type and measurements.
3. Select or clear the test with reference mic checkbox and configure parameters: dut and ref mapping, equalization file path.
4. Configure AO source settings and AI settings.
5. Set Stimulus, Case Settings, and Test Limits.
6. Click Add to test list to add the configurations.
Then you can run the test. Open the Execution pane, choose the LabVIEW version you use, click Test UUTs to run tests. Modify report path if needed. If there is UUT Information box, enter serial numbers of all your DUTs. During tests, if you want to check results, select Real-Time Result Display. After tests, if you want to check results, configure report, serial number, test list, and test case to filter. You can also go to report path directly to check results. If you want to check TestStand reports after tests, click Report.